Title:  WIRELIST FOR VCF-ELF: A 40th anniversary COSMAC ELF

Date:  August 30, 2016

File:  BUILD-WIRELIST.DOC

Revision:

Developer:				Editor:

TMSI c/o Lee Hart		Chuck Yakym,  The-Eagle@att.net

814 8th Ave N

Sartell MN 56377

leeahart@earthlink.net

______________________________________________________________________

BASIC ELF WIRING

P1 POWER/SERIAL CONNECTIONS

 WIRE SIGNAL NAME = N00046

( )  FROM PIN 8 OF P1 TO PIN ANODE OF D1

( )  FROM PIN ANODE OF D1 TO PIN ANODE OF D3

 WIRE SIGNAL NAME = N00058

( )  FROM PIN 9 OF P1 TO PIN ANODE OF D2

( )  FROM PIN ANODE OF D2 TO PIN ANODE OF D4

 WIRE SIGNAL NAME = N00060

( )  FROM PIN CATHODE OF D3 TO PIN CATHODE OF D4

( )  FROM PIN CATHODE OF D4 TO PIN COM OF S13

 WIRE SIGNAL NAME = VBAT

( )  FROM PIN CATHODE OF D1 TO PIN CATHODE OF D2

( )  FROM PIN CATHODE OF D2 TO PIN 1 OF C1

( )  FROM PIN 1 OF C1 TO PIN 22 OF IC2

( )  FROM PIN 22 OF IC2 TO PIN 22 OF IC3

( )  FROM PIN 22 OF IC3 TO PIN NO OF S3

 WIRE SIGNAL NAME = N00056

( )  FROM PIN 2 OF R4 TO PIN 4 OF P1

 WIRE SIGNAL NAME = N00057

( )  FROM PIN CATHODE OF D5 TO PIN 6 OF P1

POWER CONNECTIONS

 WIRE SIGNAL NAME = VDD

( )  FROM PIN NO OF S13 TO PIN 1 OF R1

( )  FROM PIN 1 OF R1 TO PIN 16 OF IC1

( )  FROM PIN 16 OF IC1 TO PIN 20 OF IC4

( )  FROM PIN 20 OF IC4 TO PIN 14 OF IC8

( )  FROM PIN 14 OF IC8 TO PIN 1 OF C5

( )  FROM PIN 1 OF C5 TO PIN 1 OF C6

( )  FROM PIN 1 OF C6 TO PIN 14 OF IC9

( )  FROM PIN 14 OF IC9 TO PIN NO OF S1

( )  FROM PIN NO OF S1 TO PIN NO OF S11

( )  FROM PIN NO OF S13 TO PIN 40 OF IC1

( )  FROM PIN 40 OF IC1 TO PIN 14 OF IC11

( )  FROM PIN 14 OF IC11 TO PIN 1 OF C2

( )  FROM PIN 1 OF C2 TO PIN 14 OF IC5

( )  FROM PIN 14 OF IC5 TO PIN 1 OF C3

( )  FROM PIN 1 OF C3 TO PIN 9 OF IC12

( )  FROM PIN 9 OF IC12 TO PIN 14 OF IC12

( )  FROM PIN 14 OF IC12 TO PIN 14 OF IC10

( )  FROM PIN 14 OF IC10 TO PIN 1 OF C4

( )  FROM PIN 1 OF C4 TO PIN NO OF S2

( )  FROM PIN NO OF S2 TO PIN NO OF S12

( )  FROM PIN NO OF S12 TO PIN NO OF S4

( )  FROM PIN NO OF S4 TO PIN NO OF S5

( )  FROM PIN NO OF S5 TO PIN NO OF S6

( )  FROM PIN NO OF S6 TO PIN NO OF S7

( )  FROM PIN NO OF S7 TO PIN NO OF S8

( )  FROM PIN NO OF S8 TO PIN NO OF S9

( )  FROM PIN NO OF S9 TO PIN NO OF S10

( )  FROM PIN NO OF S10 TO PIN NO OF S11

 WIRE SIGNAL NAME = VSS

( )  FROM PIN 11 OF P1 TO PIN 12 OF P1

( )  FROM PIN 12 OF P1 TO PIN 8 OF IC3

( )  FROM PIN 8 OF IC3 TO PIN 1 OF R2

( )  FROM PIN 1 OF R2 TO PIN 20 OF IC1

( )  FROM PIN 20 OF IC1 TO PIN 1 OF IC4

( )  FROM PIN 1 OF IC4 TO PIN 10 OF 1C4

( )  FROM PIN 10 OF IC4 TO PIN 7 OF IC8

( )  FROM PIN 7 OF IC8 TO PIN 2 OF C5

( )  FROM PIN 2 OF C5 TO PIN 2 OF C6

( )  FROM PIN 2 OF C6 TO PIN 7 OF IC9

( )  FROM PIN 7 OF IC9 TO PIN NC OF S1

( )  FROM PIN NC OF S1 TO PIN NC OF S11

( )  FROM PIN 11 OF P1 TO PIN 8 OF IC2

( )  FROM PIN 8 OF IC2 TO PIN 2 OF XTAL

( )  FROM PIN 2 OF XTAL TO PIN 2 OF C1

( )  FROM PIN 2 OF C1 TO PIN 7 OF IC11

( )  FROM PIN 7 OF IC11 TO PIN 2 OF C2

( )  FROM PIN 2 OF C2 TO PIN 7 OF IC5

( )  FROM PIN 7 OF IC5 TO PIN 2 OF C3

( )  FROM PIN 2 OF C3 TO PIN 8 OF IC12

( )  FROM PIN 8 OF IC12 TO PIN 7 OF IC12

( )  FROM PIN 7 OF IC12 TO PIN 6 OF IC12

( )  FROM PIN 6 OF IC12 TO PIN 4 OF IC12

( )  FROM PIN 4 OF IC12 TO PIN 7 OF IC10

( )  FROM PIN 7 OF IC10 TO PIN 2 OF C4

( )  FROM PIN 2 OF C4 TO PIN NC OF S2

( )  FROM PIN NC OF S2 TO PIN NC OF S12

( )  FROM PIN NC OF S12 TO PIN NC OF S4

( )  FROM PIN NC OF S4 TO PIN NC OF S5

( )  FROM PIN NC OF S5 TO PIN NC OF S6

( )  FROM PIN NC OF S6 TO PIN NC OF S7

( )  FROM PIN NC OF S7 TO PIN NC OF S8

( )  FROM PIN NC OF S8 TO PIN NC OF S9

( )  FROM PIN NC OF S9 TO PIN NC OF S10

( )  FROM PIN NC OF S10 TO PIN NC OF S11

1802 CLOCK CIRCUIT

 WIRE SIGNAL NAME = N00001

( )  FROM PIN 13 OF IC11 TO PIN 1 OF R3

( )  FROM PIN 1 OF R3 TO PIN 1 OF XTAL

 WIRE SIGNAL NAME = N00002

( )  FROM PIN 1 OF IC1 TO PIN 12 OF IC11

( )  FROM PIN 12 OF IC11 TO PIN 3 OF R3

 WIRE SIGNAL NAME = N00015

( )  FROM PIN 2 OF R3 TO PIN 3 OF XTAL

Q LINE CIRCUIT

 WIRE SIGNAL NAME = Q

( )  FROM PIN 1 OF R4 TO PIN 11 OF IC5

( )  FROM PIN 1 OF R4 TO PIN ANODE OF LED8

 WIRE SIGNAL NAME = N00033

( )  FROM PIN 2 OF R2 TO PIN CATHODE OF LED8

 WIRE SIGNAL NAME = N00032

( )  FROM PIN 12 OF IC5 TO PIN 13 OF IC5

( )  FROM PIN 13 OF IC5 TO PIN 4 OF IC1

/CLR CIRCUIT

 WIRE SIGNAL NAME = RUN

( )  FROM PIN COM OF S1 TO PIN 1 OF IC5

( )  FROM PIN 1 OF IC5 TO PIN 2 OF IC5

( )  FROM PIN 2 OF IC5 TO PIN 3 OF IC5

( )  FROM PIN 3 OF IC5 TO PIN 3 OF IC1

/WAIT CIRCUIT

 WIRE SIGNAL NAME = LOAD

( )  FROM PIN COM OF S2 TO PIN 9 OF IC5

( )  FROM PIN 9 OF IC5 TO PIN 5 OF IC11

( )  FROM PIN 5 OF IC11 TO PIN 4 OF IC11

 WIRE SIGNAL NAME = BARLOAD aka LOAD

( )  FROM PIN 5 OF IC5 TO PIN 3 OF IC11

( )  FROM PIN 3 OF IC11 TO PIN 6 OF IC11

( )  FROM PIN 6 OF IC11 TO PIN 2 OF IC1

A15 CIRCUIT

 WIRE SIGNAL NAME = TPA

( )  FROM PIN 3 OF IC12 TO PIN 34 OF IC1

 WIRE SIGNAL NAME = A15

( )  FROM PIN 19 OF IC3 TO PIN 19 OF IC2

( )  FROM PIN 19 OF IC2 TO PIN 1 OF IC12

 WIRE SIGNAL NAME = SC1

( )  FROM PIN 5 OF IC1 TO PIN 6 OF IC5

 WIRE SIGNAL NAME = BARINT aka INT

( )  FROM PIN 6 OF R1 TO PIN 36 OF IC1

 WIRE SIGNAL NAME = TPB

( )  FROM PIN 33 OF IC1 TO PIN 8 OF IC10

 WIRE SIGNAL NAME = BARMRD aka MRD

( )  FROM PIN 7 OF IC1 TO PIN 3 OF IC10

( )  FROM PIN 3 OF IC10 TO PIN 4 OF IC10

( )  FROM PIN 4 OF IC10 TO PIN 5 OF IC10

( )  FROM PIN 5 OF IC10 TO PIN 11 OF IC10

( )  FROM PIN 7 OF IC1 TO PIN 18 OF IC2

( )  FROM PIN 18 OF IC2 TO PIN 18 OF IC3

 WIRE SIGNAL NAME = BARMWE aka MWE

( )  FROM PIN 20 OF IC2 TO PIN 20 OF IC3

( )  FROM PIN 20 OF IC3 TO PIN 10 OF R1

( )  FROM PIN 10 OF R1 TO PIN COM OF S3

MEMORY PROTECT SWITCH

 WIRE SIGNAL NAME = N00030

( )  FROM PIN 35 OF IC1 TO PIN NC OF S3

 WIRE SIGNAL NAME = BAREF4 aka EF4

( )  FROM PIN 5 OF P1 TO PIN 2 OF R1

( )  FROM PIN 2 OF R1 TO PIN 21 OF IC1

 WIRE SIGNAL NAME = BAREF3 aka EF3

( )  FROM PIN 3 OF R1 TO PIN 22 OF IC1

( )  FROM PIN 22 OF IC1 TO PIN ANODE OF D5

 WIRE SIGNAL NAME = BAREF2 aka /EF2

( )  FROM PIN 4 OF R1 TO PIN 23 OF IC1

 WIRE SIGNAL NAME = BAREF1 aka /EF1

( )  FROM PIN 5 OF R1 TO PIN 24 OF IC1

 WIRE SIGNAL NAME = BARDMAOUT aka /DMAOUT

( )  FROM PIN 7 OF R1 TO PIN 37 OF IC1

 WIRE SIGNAL NAME = BARDMAIN aka /DMAIN

( )  FROM PIN 38 OF IC1 TO PIN 8 OF R1

( )  FROM PIN 8 OF R1 TO PIN 1 OF P1

 WIRE SIGNAL NAME = N2

( )  FROM PIN 17 OF IC1 TO PIN 8 OF IC5

 WIRE SIGNAL NAME = IN

( )  FROM PIN 7 OF P1 TO PIN 11 OF IC12

( )  FROM PIN 11 OF IC12 TO PIN 11 OF IC11

( )  FROM PIN 11 OF IC11 TO PIN 8 OF IC11

 WIRE SIGNAL NAME = BARDMA aka /DMA

( )  FROM PIN 3 OF P1 TO PIN 12 OF IC12

 WIRE SIGNAL NAME = N00059

( )  FROM PIN 10 OF IC11 TO PIN 9 OF IC11

( )  FROM PIN 9 OF IC11 TO PIN COM OF S12

 WIRE SIGNAL NAME = N00047

( )  FROM PIN 4 OF IC5 TO PIN 10 OF IC12

 WIRE SIGNAL NAME = BARDIS aka /DIS

( )  FROM PIN 17 OF IC2 TO PIN 17 OF IC3

( )  FROM PIN 17 OF IC3 TO PIN 9 OF R1

N2+LOAD CONNECTIONS

 WIRE SIGNAL NAME = N00031

( )  FROM PIN 11 OF IC4 TO PIN 9 OF IC10

 WIRE SIGNAL NAME = N00042

( )  FROM PIN 2 OF IC10 TO PIN 6 OF IC10

 WIRE SIGNAL NAME = N00043

( )  FROM PIN 1 OF IC10 TO PIN 13 OF IC10

( )  FROM PIN 13 OF IC10 TO PIN 12 OF IC10

( )  FROM PIN 12 OF IC10 TO PIN 10 OF IC5

 WIRE SIGNAL NAME = N00044

( )  FROM PIN 10 OF IC10 TO PIN 1 OF IC11

 WIRE SIGNAL NAME = N00045

( )  FROM PIN 2 OF IC11 TO PIN 13 OF IC8

( )  FROM PIN 13 OF IC8 TO PIN 12 OF IC8

( )  FROM PIN 12 OF IC8 TO PIN 5 OF IC8

( )  FROM PIN 5 OF IC8 TO PIN 6 OF IC8

( )  FROM PIN 6 OF IC8 TO PIN 13 OF IC9

( )  FROM PIN 13 OF IC9 TO PIN 12 OF IC9

( )  FROM PIN 12 OF IC9 TO PIN 5 OF IC9

( )  FROM PIN 5 OF IC9 TO PIN 6 OF IC9

ADDRESS LINES

 WIRE SIGNAL NAME = A0

( )  FROM PIN 25 OF IC1 TO PIN 4 OF IC3

( )  FROM PIN 4 OF IC3 TO PIN 4 OF IC2

 WIRE SIGNAL NAME = A1

( )  FROM PIN 26 OF IC1 TO PIN 3 OF IC3

( )  FROM PIN 3 OF IC3 TO PIN 3 OF IC2

 WIRE SIGNAL NAME = A2

( )  FROM PIN 27 OF IC1 TO PIN 2 OF IC3

( )  FROM PIN 2 OF IC3 TO PIN 2 OF IC2

 WIRE SIGNAL NAME = A3

( )  FROM PIN 28 OF IC1 TO PIN 1 OF IC3

( )  FROM PIN 1 OF IC3 TO PIN 1 OF IC2

 WIRE SIGNAL NAME = A4

( )  FROM PIN 29 OF IC1 TO PIN 21 OF IC3

( )  FROM PIN 21 OF IC3 TO PIN 21 OF IC2

 WIRE SIGNAL NAME = A5

( )  FROM PIN 30 OF IC1 TO PIN 5 OF IC3

( )  FROM PIN 5 OF IC3 TO PIN 5 OF IC2

 WIRE SIGNAL NAME = A6

( )  FROM PIN 31 OF IC1 TO PIN 6 OF IC3

( )  FROM PIN 6 OF IC3 TO PIN 6 OF IC2

 WIRE SIGNAL NAME = A7

( )  FROM PIN 32 OF IC1 TO PIN 7 OF IC3

( )  FROM PIN 7 OF IC3 TO PIN 7 OF IC2

( )  FROM PIN 32 OF IC1 TO PIN 5 OF IC12

DATA LINES

 WIRE SIGNAL NAME = D0

( )  FROM PIN 1 OF IC9 TO PIN 3 OF IC4

( )  FROM PIN 3 OF IC4 TO PIN 15 OF IC1

( )  FROM PIN 15 OF IC1 TO PIN 10 OF IC3

( )  FROM PIN 10 OF IC3 TO PIN 9 OF IC3

 WIRE SIGNAL NAME = D1

( )  FROM PIN 4 OF IC9 TO PIN 4 OF IC4

( )  FROM PIN 4 OF IC4 TO PIN 14 OF IC1

( )  FROM PIN 14 OF IC1 TO PIN 11 OF IC3

( )  FROM PIN 11 OF IC3 TO PIN 12 OF IC3

 WIRE SIGNAL NAME = D2

( )  FROM PIN 8 OF IC9 TO PIN 7 OF IC4

( )  FROM PIN 7 OF IC4 TO PIN 13 OF IC1

( )  FROM PIN 13 OF IC1 TO PIN 13 OF IC3

( )  FROM PIN 13 OF IC3 TO PIN 14 OF IC3

 WIRE SIGNAL NAME = D3

( )  FROM PIN 11 OF IC9 TO PIN 8 OF IC4

( )  FROM PIN 8 OF IC4 TO PIN 12 OF IC1

( )  FROM PIN 12 OF IC1 TO PIN 15 OF IC3

( )  FROM PIN 15 OF IC3 TO PIN 16 OF IC3

 WIRE SIGNAL NAME = D4

( )  FROM PIN 1 OF IC8 TO PIN 13 OF IC4

( )  FROM PIN 13 OF IC4 TO PIN 11 OF IC1

( )  FROM PIN 11 OF IC1 TO PIN 9 OF IC2

( )  FROM PIN 9 OF IC2 TO PIN 10 OF IC2

 WIRE SIGNAL NAME = D5

( )  FROM PIN 4 OF IC8 TO PIN 14 OF IC4

( )  FROM PIN 14 OF IC4 TO PIN 10 OF IC1

( )  FROM PIN 10 OF IC1 TO PIN 11 OF IC2

( )  FROM PIN 11 OF IC2 TO PIN 12 OF IC2

 WIRE SIGNAL NAME = D6

( )  FROM PIN 8 OF IC8 TO PIN 17 OF IC4

( )  FROM PIN 17 OF IC4 TO PIN 9 OF IC1

( )  FROM PIN 9 OF IC1 TO PIN 14 OF IC2

( )  FROM PIN 14 OF IC2 TO PIN 13 OF IC2

 WIRE SIGNAL NAME = D7

( )  FROM PIN 11 OF IC8 TO PIN 18 OF IC4

( )  FROM PIN 18 OF IC4 TO PIN 8 OF IC1

( )  FROM PIN 8 OF IC1 TO PIN 16 OF IC2

( )  FROM PIN 16 OF IC2 TO PIN 15 OF IC2

LED’S CONNECTIONS

 WIRE SIGNAL NAME = N00022

( )  FROM PIN 19 OF IC4 TO PIN ANODE OF LED7

 WIRE SIGNAL NAME = N00023

( )  FROM PIN 16 OF IC4 TO PIN ANODE OF LED6

 WIRE SIGNAL NAME = N00024

( )  FROM PIN 15 OF IC4 TO PIN ANODE OF LED5

 WIRE SIGNAL NAME = N00025

( )  FROM PIN 12 OF IC4 TO PIN ANODE OF LED4

 WIRE SIGNAL NAME = N00026

( )  FROM PIN 9 OF IC4 TO PIN ANODE OF LED3

 WIRE SIGNAL NAME = N00027

( )  FROM PIN 6 OF IC4 TO PIN ANODE OF LED2

 WIRE SIGNAL NAME = N00028

( )  FROM PIN 5 OF IC4 TO PIN ANODE OF LED1

 WIRE SIGNAL NAME = N00029

( )  FROM PIN 2 OF IC4 TO PIN ANODE OF LED0

R2 CURRENT LIMITING RESISTER SIP

 WIRE SIGNAL NAME = N00034

( )  FROM PIN 3 OF R2 TO PIN CATHODE OF LED7

 WIRE SIGNAL NAME = N00035

( )  FROM PIN 4 OF R2 TO PIN CATHODE OF LED6

 WIRE SIGNAL NAME = N00036

( )  FROM PIN 5 OF R2 TO PIN CATHODE OF LED5

 WIRE SIGNAL NAME = N00037

( )  FROM PIN 6 OF R2 TO PIN CATHODE OF LED4

 WIRE SIGNAL NAME = N00038

( )  FROM PIN 7 OF R2 TO PIN CATHODE OF LED3

 WIRE SIGNAL NAME = N00039

( )  FROM PIN 8 OF R2 TO PIN CATHODE OF LED2

 WIRE SIGNAL NAME = N00040

( )  FROM PIN 9 OF R2 TO PIN CATHODE OF LED1

 WIRE SIGNAL NAME = N00041

( )  FROM PIN 10 OF R2 TO PIN CATHODE OF LED0

DATA INPUT SWITCH CONNECTIONS

 WIRE SIGNAL NAME = N00048

( )  FROM PIN 10 OF IC8 TO PIN COM OF S4

 WIRE SIGNAL NAME = N00049

( )  FROM PIN 9 OF IC8 TO PIN COM OF S5

 WIRE SIGNAL NAME = N00050

( )  FROM PIN 3 OF IC8 TO PIN COM OF S6

 WIRE SIGNAL NAME = N00051

( )  FROM PIN 2 OF IC8 TO PIN COM OF S7

 WIRE SIGNAL NAME = N00052

( )  FROM PIN 10 OF IC9 TO PIN COM OF S8

 WIRE SIGNAL NAME = N00053

( )  FROM PIN 9 OF IC9 TO PIN COM OF S9

 WIRE SIGNAL NAME = N00054

( )  FROM PIN 3 OF IC9 TO PIN COM OF S10

 WIRE SIGNAL NAME = N00055

( )  FROM PIN 2 OF IC9 TO PIN COM OF S11

END OF BASIC ELF CONSTRUCTION WIRING, BELOW ARE THE CONNECTIONS NEEDED FOR THE OPTIONAL P2 EXPANSION PORT AND THE OPTIONAL HEX TIL311 DISPLAYS

OPTIONAL TIL311 DISPLAY CONNECTIONS (IC6, IC7)

 WIRE SIGNAL NAME = VDD

( )  FROM PIN 20 OF IC4 TO PIN 14 OF IC7

( )  FROM PIN 14 OF IC7 TO PIN 1 OF IC7

( )  FROM PIN 1 OF IC7 TO PIN 14 OF IC6

( )  FROM PIN 14 OF IC6 TO PIN 1 OF IC6

 WIRE SIGNAL NAME = VSS

( )  FROM PIN 7 OF IC5 TO PIN 5 OF IC6

( )  FROM PIN 5 OF IC6 TO PIN 7 OF IC6

( )  FROM PIN 7 OF IC6 TO PIN 8 OF IC6

( )  FROM PIN 8 OF IC6 TO PIN 7 OF IC7

( )  FROM PIN 7 OF IC7 TO PIN 5 OF IC7

( )  FROM PIN 5 OF IC7 TO PIN 8 OF IC7

 TIL311 DATA LINES

( )  FROM PIN 19 OF IC4 TO PIN 12 OF IC6

( )  FROM PIN 16 OF IC4 TO PIN 13 OF IC6

( )  FROM PIN 15 OF IC4 TO PIN 2 OF IC6

( )  FROM PIN 12 OF IC4 TO PIN 3 OF IC6

( )  FROM PIN 9 OF IC4 TO PIN 12 OF IC7

( )  FROM PIN 6 OF IC4 TO PIN 13 OF IC7

( )  FROM PIN 5 OF IC4 TO PIN 2 OF IC7

( )  FROM PIN 2 OF IC4 TO PIN 3 OF IC7

OPTIONAL P2 CONNECTIONS

 WIRE SIGNAL NAME = VDD

( )  FROM PIN 40 OF IC1 TO PIN 2 OF P2

 WIRE SIGNAL NAME = VSS

( )  FROM PIN 8 OF IC3 TO PIN 39 OF P2

 WIRE SIGNAL NAME = BARXTAL aka /XTAL

( )  FROM PIN 39 OF IC1 TO PIN 4 OF P2

 WIRE SIGNAL NAME = Q LINE

( )  FROM PIN ANODE OF LED8 TO PIN 7 OF P2

 WIRE SIGNAL NAME = RUN

( )  FROM PIN COM OF S1 TO PIN 5 OF P2

 WIRE SIGNAL NAME = BARLOAD aka LOAD

( )  FROM PIN 5 OF IC5 TO PIN 3 OF P2

 WIRE SIGNAL NAME = BARA15 aka /A15

( )  FROM PIN 2 OF IC12 TO PIN 1 OF P2

 WIRE SIGNAL NAME = TPA

( )  FROM PIN 3 OF IC12 TO PIN 14 OF P2

 WIRE SIGNAL NAME = SC0

( )  FROM PIN 6 OF IC1 TO PIN 11 OF P2

 WIRE SIGNAL NAME = SC1

( )  FROM PIN 6 OF IC5 TO PIN 9 OF P2

 WIRE SIGNAL NAME = BARINT aka INT

( )  FROM PIN 36 OF IC1 TO PIN 10 OF P2

 WIRE SIGNAL NAME = TPB

( )  FROM PIN 8 OF IC10 TO PIN 16 OF P2

 WIRE SIGNAL NAME = BARMRD aka MRD

( )  FROM PIN 7 OF IC1 TO PIN 13 OF P2

 WIRE SIGNAL NAME = BARMWE aka MWE

( )  FROM PIN 20 OF IC2 TO PIN 12 OF P2

 WIRE SIGNAL NAME = BAREF4 aka EF4

( )  FROM PIN 21 OF IC1 TO PIN 40 OF P2

 WIRE SIGNAL NAME = BAREF3 aka EF3

( )  FROM PIN 22 OF IC1 TO PIN 38 OF P2

 WIRE SIGNAL NAME = BAREF2 aka /EF2

( )  FROM PIN 23 OF IC1 TO PIN 36 OF P2

 WIRE SIGNAL NAME = BAREF1 aka /EF1

( )  FROM PIN 24 OF IC1 TO PIN 34 OF P2

 WIRE SIGNAL NAME = BARDMAOUT aka /DMAOUT

( )  FROM PIN 37 OF IC1 TO PIN 8 OF P2

 WIRE SIGNAL NAME = BARDMAIN aka /DMAIN

( )  FROM PIN 38 OF IC1 TO PIN 6 OF P2

 WIRE SIGNAL NAME = N2

( )  FROM PIN 8 OF IC5 TO PIN 33 OF P2

 WIRE SIGNAL NAME = N1

( )  FROM PIN 18 OF IC1 TO PIN 35 OF P2

 WIRE SIGNAL NAME = N0

( )  FROM PIN 19 OF IC1 TO PIN 37 OF P2

 WIRE SIGNAL NAME = BARDIS aka /DIS

( )  FROM PIN 17 OF IC2 TO PIN 31 OF P2

ADDRESS LINES

 WIRE SIGNAL NAME = A0

( )  FROM PIN 25 OF IC1 TO PIN 32 OF P2

 WIRE SIGNAL NAME = A1

( )  FROM PIN 26 OF IC1 TO PIN 30 OF P2

 WIRE SIGNAL NAME = A2

( )  FROM PIN 27 OF IC1 TO PIN 28 OF P2

 WIRE SIGNAL NAME = A3

( )  FROM PIN 28 OF IC1 TO PIN 26 OF P2

 WIRE SIGNAL NAME = A4

( )  FROM PIN 29 OF IC1 TO PIN 24 OF P2

 WIRE SIGNAL NAME = A5

( )  FROM PIN 30 OF IC1 TO PIN 22 OF P2

 WIRE SIGNAL NAME = A6

( )  FROM PIN 31 OF IC1 TO PIN 20 OF P2

 WIRE SIGNAL NAME = A7

( )  FROM PIN 32 OF IC1 TO PIN 18 OF P2

 WIRE SIGNAL NAME = D0

( )  FROM PIN 9 OF IC3 TO PIN 29 OF P2

 WIRE SIGNAL NAME = D1

( )  FROM PIN 11 OF IC3 TO PIN 27 OF P2

 WIRE SIGNAL NAME = D2

( )  FROM PIN 13 OF IC3 TO PIN 25 OF P2

 WIRE SIGNAL NAME = D3

( )  FROM PIN 15 OF IC3 TO PIN 23 OF P2

 WIRE SIGNAL NAME = D4

( )  FROM PIN 9 OF IC2 TO PIN 21 OF P2

 WIRE SIGNAL NAME = D5

( )  FROM PIN 11 OF IC2 TO PIN 19 OF P2

 WIRE SIGNAL NAME = D6

( )  FROM PIN 13 OF IC2 TO PIN 17 OF P2

 WIRE SIGNAL NAME = D7

( )  FROM PIN 15 OF IC2 PIN 15 OF P2